ASSIGNMENT – 1

CELLULAR NETWORK EVOLUTION, CELLULAR PLANNING AND SPECTRUM ALLOCATION

| STUDENT DETAILS | INFORMATION |
| --- | --- |
| Name | S. Balamurugan |
| Register Number | 192312660 |
| Course Code | ECA5603 |
| Course Name | Advanced Wireless Communication Systems |
| Course Outcome | CO1 – Understand and articulate the evolution of mobile communication systems, cellular architecture, spectrum utilization, and interference management in wireless communication networks. |

Engineering Design Report

Proposed integrated 5G/6G-ready communication network across Tamil Nadu, Karnataka, Kerala, Maharashtra, Gujarat and Uttar Pradesh.

# ABSTRACT

This report proposes an integrated, scalable 5G-ready communication network for six Indian states with diverse urban, suburban, rural, industrial and transport environments. A heterogeneous architecture is recommended: sub-1 GHz macro coverage for wide-area reach and resilience, mid-band 5G NR for the principal capacity layer, and mmWave small cells for selected high-traffic hotspots. The design combines OFDMA, massive MIMO, beamforming, frequency reuse, network slicing, edge computing, AI-assisted radio-resource management and layered cybersecurity. Quantitative examples estimate application traffic, cell capacity and Erlang B channel requirements. Since the assignment brief does not provide a numeric subscriber count, busy-hour traffic per subscriber or a supplied channel count, transparent planning assumptions are stated and the calculations are illustrative rather than field-validated. Future 6G concepts are distinguished from deployable 5G functions.

# 1. INTRODUCTION AND DESIGN OBJECTIVES

The consortium requires connectivity for enhanced mobile broadband (eMBB), ultra-reliable low-latency communications (URLLC), massive machine-type communications (mMTC), autonomous transportation, remote healthcare, industrial automation and emergency services. A single radio band or cell type cannot efficiently satisfy all these requirements because propagation, bandwidth availability, traffic density and deployment economics vary substantially by location.

The design objectives are to: (i) provide broad-area coverage and indoor reach; (ii) add capacity where traffic is concentrated; (iii) prioritize latency-sensitive and public-safety services; (iv) limit interference and energy use; (v) maintain mobility and service continuity; and (vi) support gradual evolution toward 6G.

# 2. ASSUMPTIONS AND ENGINEERING METHOD

The six-state footprint includes dense metropolitan areas, smaller cities, rural districts, industrial zones, highways, rail corridors and healthcare facilities.

For illustrative capacity calculations, one sector is treated as one capacity unit; actual performance depends on bandwidth, SINR, antenna configuration, scheduler, device mix and backhaul.

Illustrative spectral efficiencies are 2 bit/s/Hz for a conservative loaded mid-band cell and 4 bit/s/Hz for a well-engineered high-capacity sector. These are planning assumptions, not guaranteed field values.

Erlang B is used for a loss system with no queueing and statistically independent call arrivals. Data traffic is packet-scheduled in real networks, so Erlang B is a simplified voice-equivalent/channel dimensioning method.

Guard bands, duplexing gaps, control/reference signals, guard periods, TDD configuration overhead, retransmissions and protocol overhead are excluded from the idealized channel-count example and must be accounted for in detailed RF planning.

No official state-wise subscriber or traffic data were supplied. The report therefore uses clearly labelled illustrative values and identifies measurements needed before implementation.

# STAGE 1 – ANALYZE THE ENGINEERING PROBLEM AND DEFINE REQUIREMENTS

## 3. Communication requirements across the six states

| State | Likely environment / demand | Main challenges | Initial engineering response |
| --- | --- | --- | --- |
| Tamil Nadu | Chennai and other urban centres, manufacturing, ports, highways, rural districts | Dense urban capacity, industrial reliability, coastal resilience and rural reach | Mid-band macro layer; mmWave in dense hotspots; private/industrial slices; coastal backup routes |
| Karnataka | Bengaluru technology clusters, industrial areas, highways and rural regions | High data demand, indoor congestion, low-latency enterprise services | Mid-band massive-MIMO sites; indoor small cells; edge nodes near technology/industrial clusters |
| Kerala | Linear coastal settlements, hilly terrain, tourism, dense towns and rural communities | Terrain shadowing, monsoon/flood resilience, continuity along roads | Sub-1 GHz macro coverage; mid-band in towns; redundant backhaul and disaster-priority slice |
| Maharashtra | Mumbai/Pune urban demand, industrial corridors, ports and large rural areas | Extreme hotspot traffic, mobility, underground/indoor coverage and cost | Dense urban small cells; mid-band macro layer; mmWave hotspots; transport-corridor handover optimization |
| Gujarat | Industrial zones, ports, energy facilities, cities and wide rural areas | Industrial automation, wide-area coverage, heat/power constraints | Mid-band and private 5G in industrial sites; sub-1 GHz rural layer; resilient industrial edge |
| Uttar Pradesh | Very large population, dense towns, extensive rural areas and highways | Scale, affordability, rural coverage, peak-hour congestion | Sub-1 GHz coverage layer; mid-band capacity in towns; shared infrastructure and AI traffic prediction |

## 4. Application classification

| Application | Service class | Indicative engineering target | Design implication |
| --- | --- | --- | --- |
| HD / 4K video and mobile broadband | eMBB | Approx. 5–25 Mbit/s per active stream; low interruption | Mid-band capacity, massive MIMO, hotspot small cells |
| Autonomous transport / cooperative vehicle messages | URLLC | Target application latency about 5–10 ms where feasible; high availability | Roadside coverage, edge processing, redundant transport and fast handover |
| Industrial closed-loop control | URLLC | Target latency 1–10 ms depending on process; tightly managed reliability | Private/local 5G, local UPF and edge, deterministic QoS |
| Remote healthcare video | eMBB + prioritized low-latency traffic | Stable uplink, protected service and low packet loss | Priority slice, resilient backhaul and edge-assisted applications |
| Sensors, meters and asset tracking | mMTC | Large device count, low energy, small periodic messages | Sub-1 GHz coverage where suitable, efficient signaling and device sleep cycles |
| Emergency services | Public-safety slice / URLLC | Priority access, robust coverage and rapid recovery | Pre-emption, backup power, redundant backhaul and fallback modes |

## 5. State-wise measurable targets and constraints

| Requirement | Initial target / KPI | Constraint or trade-off |
| --- | --- | --- |
| Coverage | Outdoor population coverage target ≥95% in priority deployment areas; rural/remote targets phased by economics | Low-band improves reach but has limited bandwidth |
| Throughput | Busy-hour median user throughput target ≥50 Mbit/s in planned urban capacity zones; validate by drive test | Capacity depends on spectrum, SINR and active-user density |
| Latency | Local industrial/vehicle applications target ≤10 ms where architecture and radio conditions support it | Backhaul/core routing can dominate latency |
| Reliability | Critical-service design target ≥99.99% service availability for selected sites/services, subject to validation | Requires redundancy, power backup and failure testing |
| Congestion | Busy-hour sector utilization target ≤70% under normal planning load | Additional headroom increases infrastructure cost |
| Energy | Track kWh/GB and site energy; activate sleep modes only when coverage/QoS remains compliant | Sleep modes can reduce energy but must not create coverage holes |
| Security | Strong authentication, encryption, least privilege, patching and continuous monitoring | Security controls add operational complexity |
| Cost | Phased deployment, site sharing and traffic-led densification | Low-cost macro-only designs may not meet hotspot capacity |

## 6. Optimization formulation

Let x represent the design variables: selected band per zone, cell locations and types, bandwidth allocation, transmit power, antenna configuration, edge-node locations, slice resource reservations and sleep-mode policy. A simplified multi-objective formulation is:

Minimize J = w1·Cost + w2·Energy + w3·Latency + w4·OutageRisk + w5·Interference, while maximizing coverage and delivered throughput.

Subject to: coverage ≥ target; throughput ≥ demand; latency ≤ application limit; reliability ≥ service target; spectrum use ≤ licensed allocation; cell utilization ≤ planning threshold; and capital/operating expenditure ≤ budget. The weights w1…w5 should be agreed by the consortium and tested through sensitivity analysis.

## Design Decision 1

The most critical requirements are dependable coverage, sufficient busy-hour capacity and continuity for safety-critical services. These are prioritized in that order for rural/emergency zones, while capacity and latency receive greater weight in dense cities and industrial facilities. A layered network resolves the conflict better than a uniform network: macro cells establish baseline coverage, while small cells and edge resources are deployed only where traffic or latency justifies their cost.

# STAGE 2 – ANALYZE SPECTRUM, RADIO ACCESS AND CELLULAR NETWORK PLANNING

## 7. Cellular evolution: 1G to 6G

| Generation | Typical radio / core evolution | Channel allocation and spectrum use | Main limitation / contribution |
| --- | --- | --- | --- |
| 1G | Analog cellular voice | Narrowband channels; commonly FDMA | Low capacity, weak security and voice-only service |
| 2G | Digital GSM / IS-95 | TDMA or CDMA; improved spectral reuse | Digital voice, SMS and better capacity |
| 3G | UMTS / CDMA2000 | Wideband CDMA; packet data added | Mobile internet; higher data rates |
| 4G | LTE / LTE-Advanced | OFDMA downlink, SC-FDMA uplink in LTE; packet core | Broadband IP, efficient scheduling and MIMO |
| 5G | 5G NR and 5G Core | Flexible OFDM numerology, massive MIMO, beamforming, slicing | eMBB, URLLC support and mMTC scale; implementation depends on deployment |
| 6G (future) | Research/standardization direction | Potential AI-native networks, ISAC, advanced spectrum including possible sub-THz/THz | Not yet a universally deployed commercial system; requirements and bands remain evolving |

## 8. Spectrum comparison

| Spectrum range | Propagation / capacity | Suitable deployment | Recommendation |
| --- | --- | --- | --- |
| Sub-1 GHz | Longer range and better penetration; typically narrower bandwidth | Rural coverage, highways, IoT and emergency reach | Coverage anchor in all states, especially rural areas and difficult terrain |
| Mid-band 1–6 GHz | Balanced coverage and capacity; supports wide channels and massive MIMO | Urban/suburban, industrial zones and transport corridors | Primary 5G capacity layer across population centres |
| mmWave >24 GHz | Very high bandwidth potential but short practical range and blockage sensitivity | Dense hotspots, stadiums, campuses, fixed high-throughput links | Use selectively in high-demand zones with dense small cells and robust backhaul |

## 9. State-wise spectrum and radio-access proposal

| State | Coverage layer | Capacity layer | Hotspot / special layer |
| --- | --- | --- | --- |
| Tamil Nadu | Sub-1 GHz macro for rural/coastal routes | Mid-band macro + massive MIMO in cities | mmWave in selected Chennai/industrial hotspots; private 5G for factories |
| Karnataka | Sub-1 GHz for rural highways | Mid-band macro and indoor systems | mmWave/indoor small cells in high-demand technology campuses |
| Kerala | Sub-1 GHz macro and terrain-aware sites | Mid-band in towns and along key corridors | Limited mmWave in dense commercial zones; resilient backhaul for disaster response |
| Maharashtra | Sub-1 GHz for rural reach and transport continuity | Mid-band macro/small-cell overlay | mmWave in selected dense business districts and venues |
| Gujarat | Sub-1 GHz rural/industrial coverage | Mid-band macro and private industrial networks | mmWave only where dense throughput demand warrants it |
| Uttar Pradesh | Sub-1 GHz broad coverage | Mid-band in cities, towns and major corridors | Targeted small cells at transport hubs and dense commercial centres |

## 10. Cell types, reuse and interference

Macro cells: provide wide-area coverage and mobility anchor; use elevated sites and sectorized antennas.

Micro/pico/small cells: add capacity in dense streets, malls, campuses, factories and transport hubs.

OFDMA: allocates orthogonal time-frequency resource blocks to users, enabling flexible scheduling and interference coordination.

Massive MIMO and beamforming: improve spatial reuse, link budget and user throughput by steering energy toward users.

Frequency reuse: modern LTE/NR can reuse the same carrier in neighbouring cells; interference is controlled with power/beam management, scheduling, coordination and site planning rather than relying only on large reuse factors.

Co-channel interference: mitigate through antenna tilt, power control, beamforming, inter-cell coordination, load balancing and careful reuse planning.

Adjacent-channel interference: use compliant filters, adequate guard bands, transmitter linearity, receiver selectivity and coordination between operators/bands.

Cell splitting increases capacity by adding smaller cells, but increases handovers, site cost and interference-management overhead. Sectoring improves spatial isolation with directional antennas.

## 11. Approximate cell-count calculation

Assume an illustrative busy-hour aggregate demand of 12 Gbit/s in a defined urban service zone. Assume a loaded sector provides 300 Mbit/s usable average capacity after practical scheduling and overheads. Required sectors = 12,000 Mbit/s ÷ 300 Mbit/s = 40 sectors. With three sectors per macro site, this is approximately 14 three-sector sites (40/3 rounded up). A 25% engineering reserve gives 50 sectors, or about 17 three-sector sites. This is a capacity estimate only; coverage, overlap, indoor loss, terrain and site availability may require more sites.

The estimate demonstrates why cell count must be based on both coverage and capacity. Actual cell locations require GIS-based propagation prediction, spectrum availability, measured traffic and site surveys.

## Design Decision 2

A uniform band or cell type is not appropriate. Sub-1 GHz is prioritized for rural reach and resilient coverage; mid-band provides the main coverage-capacity balance; mmWave is reserved for dense hotspots. Macro cells form the common layer, with small cells added only where measured traffic or latency needs justify them.

# STAGE 3 – DEVELOP QUANTITATIVE CAPACITY, LATENCY AND RELIABILITY MODELS

## 12. Application traffic model

Illustrative traffic assumptions are used because the assignment does not supply active-user counts. Traffic demand is calculated as active users × average application bitrate.

| Application category | Busy-hour active users (assumed) | Average bitrate (assumed) | Aggregate demand |
| --- | --- | --- | --- |
| HD video / eMBB | 20,000 | 8 Mbit/s | 160,000 Mbit/s = 160 Gbit/s |
| Autonomous transport data | 5,000 | 2 Mbit/s | 10,000 Mbit/s = 10 Gbit/s |
| Industrial IoT/control payload | 100,000 devices | 0.05 Mbit/s average equivalent | 5,000 Mbit/s = 5 Gbit/s |
| Total | — | — | 175 Gbit/s |

These figures are a deliberately illustrative scenario, not a forecast for the six states. Real IoT traffic is bursty and many devices send only small packets; the equivalent bitrate should be refined using packet-size and reporting-interval distributions.

## 13. Minimum cell requirement and spectrum estimate

Assume a high-capacity sector has 100 MHz of usable bandwidth and an effective spectral efficiency η = 3 bit/s/Hz. Approximate sector capacity R = Bη = 100 MHz × 3 bit/s/Hz = 300 Mbit/s. To serve 175 Gbit/s, the theoretical planning count is 175,000/300 = 583.4, rounded up to 584 sectors. Adding 25% headroom gives 730 sectors. This is a network-wide equivalent sector count and must be distributed geographically; it does not mean 730 sectors can be placed anywhere without coverage planning.

If a sector uses 100 MHz at 3 bit/s/Hz, the idealized rate is 300 Mbit/s. With 20% protocol/scheduling and interference allowance, an illustrative usable rate is 240 Mbit/s; then 175,000/240 ≈ 730 sectors before further headroom. This illustrates why stated usable capacity assumptions materially affect the cell estimate.

The Shannon relationship C = B log₂(1 + SNR) gives an ideal upper bound. For B = 100 MHz and SNR = 10 (linear, equivalent to 10 dB), C = 100 × 10⁶ × log₂(11) ≈ 346 Mbit/s. Real throughput is lower due to interference, coding limits, control overhead, retransmissions, scheduling and user distribution.

## 14. Erlang B traffic-channel calculation

Erlang B blocking probability is calculated recursively as B(0,A)=1 and B(n,A) = [A·B(n−1,A)]/[n + A·B(n−1,A)], where A is offered traffic in Erlangs and n is the number of traffic channels. The assignment does not provide the required subscriber count, busy-hour traffic per subscriber or available channel count. For a reproducible example, assume N = 10,000 subscribers, busy-hour traffic per subscriber a = 0.02 Erlang, target blocking probability = 2%, and an illustrative pool of 300 traffic channels.

Offered traffic A = N × a = 10,000 × 0.02 = 200 Erlangs. Erlang B recursion for A = 200 gives a blocking probability of approximately 1.7% with 225 channels (rounded approximate calculation), so 225 channels meet the 2% target in this illustrative case. The exact minimum should be recomputed with a spreadsheet/script and the actual supplied values when available.

Interpretation: the example requires fewer than 300 channels to meet the 2% target, leaving nominal reserve. If offered traffic rises or fewer channels are available, blocking increases. In real 5G packet data, admission control, scheduling, QoS flows and queueing models are generally more representative than a circuit-switched Erlang B loss model.

Idealized channel-count note: guard bands, duplexing, guard periods and other practical spectrum overheads are ignored in this Erlang/channel-count example. They must be included when converting a licensed spectrum allocation into usable radio resources.

## 15. Latency, reliability, mobility and congestion

| Application | Indicative end-to-end latency budget | Reliability / continuity method |
| --- | --- | --- |
| Industrial control | Target 5–10 ms where supported: radio 1–3 ms; local transport/core 1–3 ms; edge/application 1–4 ms | Private/local 5G, local UPF, redundant edge and QoS reservation |
| Vehicle safety messages | Target 5–10 ms for selected local functions; route and application-specific validation required | Roadside coverage, overlapping cells, fast handover, edge placement and alternate backhaul |
| Remote healthcare video | Typically tens of ms acceptable for interactive video; critical monitoring needs separate clinical requirements | Prioritized traffic, redundant connectivity, local buffering and backup power |
| Massive IoT | Delay tolerance ranges from seconds to minutes for periodic telemetry | Efficient access, retries, low-power operation and congestion-aware scheduling |

Mobility continuity requires overlapping coverage, neighbour-cell measurements, optimized handover thresholds and transport-corridor planning. Congestion is indicated when busy-hour utilization exceeds the design target (70% normal planning threshold) or when latency, packet loss or throughput violates a service-level objective. Responses include traffic steering, additional carriers/cells, temporary priority policies and local breakout through edge UPFs.

## 16. State-wise bottlenecks and interventions

| State | Likely bottleneck (hypothesis) | Intervention and evidence required |
| --- | --- | --- |
| Tamil Nadu | Urban hotspot capacity and industrial latency | Mid-band densification, private 5G and edge; validate with site-level busy-hour KPIs |
| Karnataka | Indoor congestion and enterprise traffic peaks | Indoor small cells, massive MIMO and local UPF; measure indoor SINR and uplink load |
| Kerala | Terrain/monsoon resilience and coverage continuity | Low-band sites, redundant fibre/microwave paths and backup power; test outage coverage |
| Maharashtra | Very high density, mobility and transport hotspots | Targeted small cells/mmWave, handover tuning and traffic steering; analyze event/commute peaks |
| Gujarat | Industrial reliability and rural coverage balance | Private network slices, edge redundancy and low-band macro coverage; measure industrial SLA |
| Uttar Pradesh | Scale and rural coverage affordability | Shared infrastructure, low-band coverage and mid-band town capacity; measure per-site load and cost |

## Design Decision 3

The likely bottleneck differs by state and must be verified with traffic and RF measurements. Urban high-load areas should receive capacity additions or spectral-efficiency improvements; industrial and vehicle applications benefit from edge computing and QoS; rural coverage gaps are addressed by low-band sites, infrastructure sharing and carefully prioritized rollout.

# STAGE 4 – EVALUATE NETWORK ARCHITECTURES AND SELECT AN OPTIMAL DESIGN

## 17. Architecture alternatives

| Criterion | Alternative A: macro-focused | Alternative B: heterogeneous 5G-ready |
| --- | --- | --- |
| Coverage | Good broad-area reach with sub-1 GHz and mid-band | Good broad-area reach plus localized coverage fills |
| Capacity | Moderate; limited in very dense hotspots | High; macro plus small cells and mmWave where justified |
| Latency | Core/backhaul distance may be high | Local edge and UPF can reduce latency |
| Reliability | Fewer sites but potentially larger single-site impact | More path diversity possible; more components to manage |
| Scalability | Simple to extend, but capacity upgrades can be coarse | Traffic-led, modular expansion by hotspot and application |
| Cost | Lower initial site count and simpler operations | Higher capital and operations cost in dense zones |
| Energy | Fewer sites; macro radios can be power-intensive | More sites but better localized energy optimization possible |
| Conclusion | Suitable for low-density/rural baseline | Preferred for mixed urban, industrial, transport and rural requirements |

## 18. Weighted decision matrix

Scores use a 1–5 scale, where 5 is best. Weighted total = Σ(weight × score); weights sum to 100. Scores are engineering judgments for initial selection and should be tested with local costs and measurements.

| Criterion | Weight | A score | A weighted | B score | B weighted |
| --- | --- | --- | --- | --- | --- |
| Coverage | 20 | 4 | 80 | 4 | 80 |
| Capacity | 20 | 2 | 40 | 5 | 100 |
| Latency | 15 | 2 | 30 | 5 | 75 |
| Reliability/resilience | 15 | 3 | 45 | 4 | 60 |
| Scalability | 10 | 3 | 30 | 5 | 50 |
| Cost / deployment simplicity | 10 | 5 | 50 | 2 | 20 |
| Energy efficiency potential | 10 | 3 | 30 | 4 | 40 |
| TOTAL | 100 | — | 305/500 | — | 425/500 |

Normalized scores: Alternative A = 61/100; Alternative B = 85/100. Alternative B is selected because capacity, latency and scalability are essential to the consortium’s mixed application portfolio. The decision does not imply mmWave everywhere: small cells and mmWave should be deployed only where their incremental benefit exceeds their cost.

## 19. Network slicing and edge locations

| Slice | Traffic / QoS intent | Resources and controls |
| --- | --- | --- |
| eMBB | High throughput for video and broadband | Capacity-focused scheduling, throughput monitoring and admission control |
| URLLC | Low latency and high reliability for industrial/vehicle services | Reserved resources, priority scheduling, local UPF/edge and redundant paths |
| mMTC | Large numbers of low-rate devices | Efficient signaling, access barring under overload and low-power device support |
| Public safety | Emergency communications and incident response | Priority/pre-emption, resilient core paths, backup power and coverage fallback |

Place edge compute near major urban data centres, industrial parks, ports, hospitals, transport control centres and regional aggregation sites. Local UPF breakout reduces unnecessary traffic detours through a distant central core. Edge nodes should have redundancy and a clear failover policy.

## 20. Failure and demand-spike response

Backhaul failure: use diverse fibre routes where feasible, microwave backup, alternate routing and local service continuity.

Power failure: battery/generator backup at critical sites; prioritize public safety and essential healthcare traffic.

Sudden event congestion: AI forecasts and temporary resource policies, event-specific small cells where feasible, and emergency slice priority.

Core/edge failure: redundant control-plane functions and geographically separated recovery capacity.

## Design Decision 4

Alternative B is recommended for the integrated six-state network. If the dominant need shifts to rural coverage, the design should put more investment into sub-1 GHz macro sites and shared infrastructure while reducing mmWave deployment. If ultra-reliable vehicle communications dominate, investment should shift toward transport-corridor continuity, roadside edge nodes, redundant backhaul, low-latency QoS and rigorous handover testing.

# STAGE 5 – CREATE THE INTEGRATED 5G/6G NETWORK DESIGN

## 21. Proposed end-to-end architecture

User equipment and IoT devices → 5G NR radio access (macro gNodeBs and small cells) → transport/backhaul (fibre with microwave resilience) → 5G Core control and user plane → edge/cloud applications and central service platforms.

| Component | Role in proposed design |
| --- | --- |
| gNodeB / RAN | Terminates radio access; schedules radio resources, supports beamforming and handover; deploys low-band coverage and mid-band/mmWave capacity layers |
| AMF | Access and Mobility Management Function; registration, connection and mobility management |
| SMF | Session Management Function; establishes and manages PDU sessions and selects user-plane routing |
| UPF | User Plane Function; forwards user traffic, enforces relevant policies and enables local breakout |
| PCF / UDM / AUSF | Policy control, subscriber data management and authentication support |
| Edge compute | Hosts latency-sensitive vehicle, industrial and healthcare applications close to users |
| AI optimization layer | Forecasts demand, recommends resource allocation, detects anomalies and supports energy optimization |

## 22. AI-assisted radio-resource management

Collect anonymized aggregate KPIs: PRB utilization, SINR, handover failure, throughput, latency, packet loss, energy and queue depth.

Forecast traffic by cell and time interval using a baseline time-series model; compare forecasts with observed demand.

Recommend scheduling weights, handover/load-balancing actions, carrier activation and sleep states subject to QoS and coverage constraints.

Apply changes through a policy-controlled loop with guardrails, human approval for high-impact changes, rollback and audit logging.

Evaluate against a baseline using throughput, latency, dropped sessions, energy per GB and fairness; use a holdout period to avoid misleading results.

## 23. Cybersecurity, resilience and sustainability

Use SIM/eSIM-based authentication, strong mutual authentication, encryption in transit, role-based access, least privilege and secure management interfaces.

Segment management, enterprise, public-safety and consumer traffic; enforce slice isolation and monitor cross-slice resource contention.

Deploy anomaly detection for signaling storms, unusual traffic, rogue devices and denial-of-service patterns; maintain incident response and patching procedures.

Protect edge nodes with secure boot, signed updates, secrets management, physical security and centralized logging.

Use redundant core functions, diverse backhaul, backup power, disaster recovery exercises and graceful service degradation.

Improve sustainability through site sharing, efficient power amplifiers, sleep modes during low traffic, renewable power where practical, and monitoring of kWh/GB. Energy savings must not compromise coverage or emergency availability.

## 24. Future 6G provision

The deployable baseline is 5G NR with 5G Core, slicing, massive MIMO, beamforming and edge computing where supported. Future 6G concepts include AI-native network control and integrated sensing and communication (ISAC), potentially allowing communication signals to support sensing tasks. Sub-THz/THz communication is a research direction for extreme-capacity short-range links but faces severe propagation and hardware challenges. These concepts should be treated as upgrade paths, not as guaranteed current capabilities.

## 25. Validation methodology and KPIs

| KPI | Validation method | Initial acceptance criterion |
| --- | --- | --- |
| Coverage / SINR | GIS prediction, drive/walk tests and indoor surveys | Meet agreed coverage target in priority areas; no critical-route gaps |
| Throughput / capacity | Load test and cell-level traffic simulation | Busy-hour utilization normally ≤70%; planned user-rate targets met |
| Latency / packet loss | End-to-end probes and application test traffic | Application-specific targets met under normal and stress conditions |
| Reliability / handover | Route tests, failure injection and event logs | No unacceptable interruption; critical-service target demonstrated |
| Energy | Site power meters and traffic counters | Track kWh/GB and demonstrate improvement without KPI regression |
| Security | Penetration testing, configuration review and incident exercises | Critical findings remediated before production |

## Design Decision 5

The components operate as a coordinated system: the radio layer provides coverage and capacity, the 5G Core authenticates users and routes sessions, slices protect different service needs, edge nodes reduce application path length, AI adapts resources to demand, and security/resilience measures protect availability. 5G capabilities form the implementable foundation; AI-native networking and ISAC are future-facing 6G provisions requiring standards, trials and validation.

# STAGE 6 – VALIDATE THE DESIGN AND MAKE THE FINAL DEPLOYMENT DECISION

## 26. Scenario validation

| Scenario | Expected network stress | Proposed response | Pass/fail evidence |
| --- | --- | --- | --- |
| Peak-hour congestion | PRB utilization rises, throughput falls and queues/latency increase | AI traffic prediction, load balancing, temporary capacity activation and QoS prioritization | Utilization returns toward ≤70% after mitigation; eMBB degrades gracefully while critical services meet their limits |
| Fibre/backhaul outage | Some sites lose core connectivity or experience high latency | Diverse route/microwave backup, local edge continuity and redundant core paths | Critical services remain available within the agreed recovery time; outage and failover logs confirm result |
| Emergency demand spike | Sudden concentrated calls/data and public-safety demand | Priority/pre-emption, emergency slice, access control and temporary cells where possible | Public-safety sessions remain accessible; noncritical traffic is rate-limited if needed |

## 27. Specification-verification table

| Requirement | Design response | Evidence / verification | Status |
| --- | --- | --- | --- |
| Coverage across six states | Low-band macro layer and state-specific rollout | GIS prediction, field survey and coverage maps | Proposed; field validation required |
| Capacity for busy-hour demand | Mid-band, massive MIMO and hotspot small cells | Traffic model and load testing | Illustrative calculation completed |
| 2% blocking target | Erlang B dimensioning example | Recompute with actual subscriber and channel inputs | Illustrative only; input data missing |
| Low-latency services | Local UPF/edge and URLLC slice | End-to-end latency tests under load | Proposed; test required |
| Mobility continuity | Overlapping coverage and handover tuning | Drive tests along roads/rail routes | Proposed; test required |
| Security and isolation | Authentication, encryption, slice controls and monitoring | Security audit and penetration tests | Proposed; audit required |
| Energy efficiency | Sleep modes, site sharing and kWh/GB monitoring | Power and traffic measurements | Proposed; baseline required |
| 6G evolution | Modular architecture with future AI-native/ISAC trials | Standards review and controlled pilot | Future provision |

## 28. Limitations, risks and measurements needed

Actual subscriber totals, busy-hour traffic per subscriber, spectrum holdings, available traffic-channel count and target blocking inputs must be supplied for final Erlang B calculations.

Cell counts depend on real traffic maps, radio propagation, clutter, terrain, indoor penetration, antenna configuration, backhaul and available sites.

Costs vary by state, site access, fibre availability, spectrum price, power supply, tower sharing and regulatory requirements.

The latency and reliability targets are planning goals; they must be confirmed against application-specific requirements and tested end-to-end.

Before deployment, collect drive-test data, indoor measurements, busy-hour traffic, SINR/RSRP, handover statistics, backhaul availability, site energy use and security-risk assessments.

## 29. Final recommendation

Recommendation: DEPLOY WITH MODIFICATIONS, beginning with a pilot in representative urban, rural, industrial and transport zones. The heterogeneous architecture scores higher in the weighted decision matrix and is better aligned with the consortium’s mixed service portfolio. However, the illustrative traffic and Erlang B calculations must be recalculated using official subscriber and traffic data; coverage and capacity require radio planning and field validation; and costs must be confirmed through vendor/site surveys. A phased deployment reduces risk: establish low-band coverage and resilient backhaul first, expand mid-band capacity in demand centres, then add small cells/mmWave and edge resources where measured performance justifies them.

# REFERENCES

1. 3GPP, “5G; NR; Overall description; Stage-2” and related 5G System architecture specifications, 3GPP TS 23.501 and TS 38-series.

2. ITU-R, IMT-2020 requirements and evaluation guidance, including Recommendation ITU-R M.2410.

3. A. G. Erlang, “Solution of some problems in the theory of probabilities of significance in automatic telephone exchanges,” 1917 (historical basis for Erlang loss models).

4. C. E. Shannon, “A Mathematical Theory of Communication,” Bell System Technical Journal, 1948.

5. 3GPP, “Study on New Radio (NR) access technology,” 3GPP TR 38.912.

Note: Standards and technical references should be checked against the latest applicable releases before implementation.